Příloha č. 1: Protokol o výpočtu GPS

| Statistics |  |
| --- | --- |
| Adjustment in the national system. |  |
| System: Czech Republic - Krovak (Datum: Krovak, Projection: Czech Republic - Krovak) |  |
| Geoidmodel | EGM96 (Global) |
| Number of baselines | 4 |
| Number of terrestrial measurements | 0 |
| Number of control points in national system | 3 |
| Number of adjusted points | 5 |
| Confidence level | 1 Sigmas |
| Significance level for tau test | 1.00 % |
| Standard error of unit weight | 2.527 |
| Number of iterations | 3 |

| ## 1. | ## Adjusted Transformation Parameters WGS84->National: |
| --- | --- |

| Parameter | Value | Std.Dev. |
| --- | --- | --- |
| Scale | 1.000003593 | 0.000008361 |
| Rotation X | 16.4405 arcsec | 5.1056 arcsec |
| Rotation Y | -3.2831 arcsec | 16.0472 arcsec |
| Rotation Z | -9.3450 arcsec | 3.9351 arcsec |
| Translation X | -650.2145 m |  |
| Translation Y | -644.8478 m |  |
| Translation Z | -353.3847 m |  |

The translation has been calculated using the WGS84 coordinates of point 00090250332.

| ## 2. | ## Baselines Input in WGS84 (Components and Std.Dev.) |
| --- | --- |

| Baseline | DX [m] | DY [m] | DZ [m] | sDX [mm] | sDY [mm] | sDZ [mm] | Solution |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00090252050-00090250332 | 121.4627 | 567.4423 | -186.9910 | 7.9 | 4.0 | 9.5 | Double Diff. / Fixed / L1 |
| 00090252050-00090252040 | 222.7777 | 513.6084 | -285.6187 | 28.6 | 16.2 | 41.2 | Double Diff. / Fixed / Ln |
| 00090252050-00090252330 | 369.4659 | -755.9747 | -269.3933 | 5.9 | 2.7 | 7.3 | Double Diff. / Fixed / L1 |
| 00090252050-00090252340 | -203.0035 | -345.4991 | 252.9394 | 18.8 | 8.6 | 21.7 | Double Diff. / Fixed / L1 |

Standard deviations of the static baselines have been multiplied with the factor 10.00.
Baselines which were rejected by the statistical test are marked.

| ## 3. | ## National Control Points Input (Plane Coord. and Std.Dev.) |
| --- | --- |

| Point | N [m] | E [m] | ell.H [m] | sN [mm] | sE [mm] | sH [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090250332 | 997571.9400 | 845235.4400 | 917.2000 | 0.0 | 0.0 | 0.0 |
| 00090252330 | 997372.5300 | 846565.7100 | 819.6100 | 0.0 | 0.0 | 0.0 |
| 00090252340 | 996771.4500 | 845936.1100 | 924.2900 | 0.0 | 0.0 | 0.0 |

Coordinates which were not considered in the adjustment are in brackets.

| ## 4. | ## Adjusted Baselines in WGS84 (Components and Std.Dev.) |
| --- | --- |

| Baseline | DX [m] | DY [m] | DZ [m] | sDX [mm] | sDY [mm] | sDZ [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090252050-00090250332 | 121.4618 | 567.4472 | -186.9974 | 19.9 | 9.4 | 23.5 |
| 00090252050-00090252040 | 222.7777 | 513.6084 | -285.6187 | 72.2 | 40.8 | 104.0 |
| 00090252050-00090252330 | 369.4658 | -755.9758 | -269.3894 | 14.9 | 6.6 | 18.0 |
| 00090252050-00090252340 | -202.9665 | -345.5157 | 252.9576 | 38.1 | 9.2 | 38.4 |

| ## 5. | ## Baseline Residuals (Residuals and Standardized Res.) |
| --- | --- |

| Baseline | vN [mm] | vE [mm] | vH [mm] | vN/svN | vE/svE | vH/svH |
| --- | --- | --- | --- | --- | --- | --- |
| 00090252050-00090250332 | -4.2 | 4.9 | -4.7 | -0.981 | 1.411 | -1.296 |
| 00090252050-00090252040 | 0.0 | 0.0 | 0.0 | 0.000 | 0.000 | 0.000 |
| 00090252050-00090252330 | 2.7 | -1.1 | 2.8 | 1.216 | -0.542 | 0.683 |
| 00090252050-00090252340 | -13.3 | -24.5 | 34.6 | -0.652 | -0.984 | 0.845 |

Baselines which were rejected by the statistical test are marked.

| ## 6. | ## Adjusted Points in WGS84 (Cart. Coordinates and Residuals) |
| --- | --- |

| Point | X [m] | Y [m] | Z [m] | resX [mm] | resY [mm] | resZ [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090250332 | 3974939.0350 | 910974.3633 | 4888950.7881 | 0.0 | 0.0 | -0.0 |
| 00090252040 | 3975040.3509 | 910920.5245 | 4888852.1667 | ------ | ------ | ------ |
| 00090252050 | 3974817.5732 | 910406.9161 | 4889137.7855 | ------ | ------ | ------ |
| 00090252330 | 3975187.0390 | 909650.9403 | 4888868.3961 | 0.0 | 0.0 | -0.0 |
| 00090252340 | 3974614.6066 | 910061.4004 | 4889390.7430 | 0.0 | 0.0 | -0.0 |

| ## 7. | ## Adjusted Points in WGS84 (Cart. Coordinates and Std.Dev.) |
| --- | --- |

| Point | X [m] | Y [m] | Z [m] | sX [mm] | sY [mm] | sZ [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090250332 | 3974939.0350 | 910974.3633 | 4888950.7881 | 0.0 | 0.0 | 0.0 |
| 00090252040 | 3975040.3509 | 910920.5245 | 4888852.1667 | 74.7 | 41.5 | 106.4 |
| 00090252050 | 3974817.5732 | 910406.9161 | 4889137.7855 | 19.3 | 7.4 | 22.3 |
| 00090252330 | 3975187.0390 | 909650.9403 | 4888868.3961 | 0.0 | 0.0 | 0.0 |
| 00090252340 | 3974614.6066 | 910061.4004 | 4889390.7430 | 0.0 | 0.0 | 0.0 |

| ## 8. | ## Adjusted Points in WGS84 (Geogr. Coordinates and Std.Dev.) |
| --- | --- |

| Point | Lat [Deg] | Lon [Deg] | ell.H [m] | sN [mm] | sE [mm] | sH [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090250332 | N 50° 21' 24.69649'' | E 12° 54' 29.17745'' | 962.9636 | 0.0 | 0.0 | 0.0 |
| 00090252040 | N 50° 21' 20.49938'' | E 12° 54' 25.37842'' | 942.3569 | 63.7 | 39.4 | 114.1 |
| 00090252050 | N 50° 21' 34.66479'' | E 12° 54' 02.57207'' | 950.5781 | 8.0 | 8.5 | 28.0 |
| 00090252330 | N 50° 21' 24.33301'' | E 12° 53' 21.12720'' | 865.2722 | 0.0 | 0.0 | 0.0 |
| 00090252340 | N 50° 21' 46.73865'' | E 12° 53' 47.82715'' | 969.9585 | 0.0 | 0.0 | 0.0 |

| ## 9. | ## Adjusted Points in National System (Plane Coord. and Residuals) |
| --- | --- |

| Point | N [m] | E [m] | ell.H [m] | resN [mm] | resE [mm] | resH [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090250332 | 997571.9393 | 845235.4397 | 917.2000 | -0.7 | -0.3 | 0.0 |
| 00090252040 | 997688.2748 | 845329.8876 | 896.5926 | ------ | ------ | ------ |
| 00090252050 | 997185.4932 | 845706.6593 | 904.8693 | ------ | ------ | ------ |
| 00090252330 | 997372.5293 | 846565.7097 | 819.6100 | -0.7 | -0.3 | -0.0 |
| 00090252340 | 996771.4493 | 845936.1097 | 924.2900 | -0.7 | -0.3 | 0.0 |

| ## 10. | ## Adjusted Points in National System (Plane Coord. and Std.Dev.) |
| --- | --- |

| Point | N [m] | E [m] | ell.H [m] | sN [mm] | sE [mm] | sH [mm] |
| --- | --- | --- | --- | --- | --- | --- |
| 00090250332 | 997571.9393 | 845235.4397 | 917.2000 | 0.0 | 0.0 | 0.0 |
| 00090252040 | 997688.2748 | 845329.8876 | 896.5926 | 63.7 | 39.4 | 114.1 |
| 00090252050 | 997185.4932 | 845706.6593 | 904.8693 | 8.0 | 8.5 | 28.0 |
| 00090252330 | 997372.5293 | 846565.7097 | 819.6100 | 0.0 | 0.0 | 0.0 |
| 00090252340 | 996771.4493 | 845936.1097 | 924.2900 | 0.0 | 0.0 | 0.0 |

| ## 11. | ## Adjusted Points Error Ellipses |
| --- | --- |

| Point | A [mm] | B [mm] | Angle [Deg] |
| --- | --- | --- | --- |
| 00090250332 | 0.0 | 0.0 | 90.0 |
| 00090252040 | 65.4 | 36.5 | -16.0 |
| 00090252050 | 9.5 | 6.7 | -49.8 |
| 00090252330 | 0.0 | 0.0 | 90.0 |
| 00090252340 | 0.0 | 0.0 | 90.0 |